# UGMFGT-15-1 Programming for Engineers

# Coursework: Power Quality Waveform Analyser

## What is the goal of the project

To build a C program that reads real world power quality data from a sensor log file, analyses the waveforms, and provides a report of all inputted data. The dataset provided is of a three phase industrial power supply at 50Hz over 200ms. This project represents a real world function of data processing that could be used to, for example, diagnose faults on an industrial site or check for grid compliance.

The data is supplied in a .csv file type and we must construct an executable file that will fetch the data from the csv, compute all required statistics, and then provide a .txt file output.

## Some context

Power quality monitoring is a core task in electrical engineering. Grid-connected systems must maintain voltage levels within defined tolerances. When voltage deviates — due to overload, switching events, or equipment faults — it can damage equipment or violate compliance standards.

This program will act as an automated analysis tool: it ingests a raw log file, computes key power quality metrics for each phase, and flags any anomalies.

## How does this translate to requirements?

| # | Requirement | What this means in practice |
| --- | --- | --- |
| 1 | Define a WaveformSample struct | Create a struct type in a header file (.h) that holds all 8 fields for a single row of the CSV |
| 2 | Dynamic memory allocation | Allocate an array of WaveformSample using malloc() sized to the number of rows in the file; free all memory before the program exits |
| 3 | Pointer-based data access | Use pointer arithmetic or pointer parameters to pass and modify data — do not rely solely on array indexing |
| 4 | File I/O to load data | Open the CSV, parse the header row, read each data row into your struct array, and handle file-not-found errors gracefully |
| 5 | Compute RMS voltage (per phase) | RMS = sqrt(mean of squared samples); compare against the 230 V nominal |
| 6 | Compute peak-to-peak amplitude | Maximum sample minus minimum sample, per phase |
| 7 | Compute DC offset | Mean voltage per phase; should be close to 0 V for a clean AC signal |
| 8 | Detect clipping | Flag any sample where ¦voltage¦ >= 324.9 V (the hard limit of the sensor) |
| 9 | Check tolerance compliance | Report whether each phase RMS is within ±10% of 230 V |
| 10 | Export results to file | Write a plain-text report to results.txt with all computed metrics |
| 11 | Modular code structure | Separate your code into at least: main.c, waveform.c, waveform.h, io.c, io.h |
| 12 | GitHub repository | Commit regularly; at least 8 meaningful commits; commit messages must describe what changed and why |

## How do these translate into an actual program?

These are the basic requirements of the project. So we can now begin to understand a logical framework of how a program of the described goal will operate:

Initial impression on logical steps to the process

- A storage array of structures will be declared.

- The CSV file will be opened.

- The data will be read one row at a time into each element of the array.

- The CSV file will be closed.

- The array of data will be passed into multiple predefined functions to compute all required output values, which will be:

  - RMS voltage for each individual phase, and check for tolerance

  - Peak to peak amplitude of each individual phase

  - The mean voltage per phase

  - Flagging of clipped values

- Write the output to a text file

On top of the functionality requirements there are also certain requirements to how we produce this outcome.

malloc()

The first of these is that malloc() must be used to allocate memory for the array dynamically based on file rows and clear this memory before exiting the program. As I was not familiar the function, research was required. I found that malloc directly works to the heap of a C program, and you use it when defining a new variable to directly state how many bits of data are needed for this variable instead of relying on the system to allocate the memory for you. The benefit of doing this is that, unlike the stack, the memory persists even after the function that allocated it has finished, so the address to the array in memory can be safely returned, as stated by GeeksforGeeks.  So in knowing this, the use of malloc has become clear, in that I will pass certain parameters into the function relating to number of rows and the size in memory of each row to define the array size from the data from the CSV file.

In this case free() is not technically needed as the same data is used throughout and the memory is automatically freed by the operating system, however I will add it for the sake of it. If the OS wasn’t capable of this, free() would be needed to prevent a memory leak from happening, where the program keeps taking up more and more memory while the system stays on until it would eventually run out and crash, hence why it is vital to be disciplined on systems that have less safeguards.

Pointers

Pointers are required to be used here to iterate through the array as we know the size of each element in the array and the length of the ray, hence pointers can be used to access the array in a safer manner as opposed to relying on the inbuilt array indexing.

Code structure

The brief also requires that the project code is laid out in a modular and readable manner. The main layout that the requirement states is the use of header (.h) and source (.c) files. To further enhance the code structure, I intend to package all headers into one folder, and the same for source files. Further inclusions will consist of the CMake.txt file, a folder for test units and the build folder produced from CMake. Further inclusions may later appear as the process unfolds.

Github repository

The project should be version controlled with Github, ensuring a reasonable level of commits and clarity in the purpose of each.

There are also extension requirements that will be resolved after the base prototype is completed.

Now that the requirements are fully understood I can move on to an initial rough plan for the project.

## Initial plan proposal

- A code structure diagram should be completed to visually represent the layout of the project code

- A logical flow of inputs and outputs between files & functions etc should be made as a visual aid to the process

- The layout should then be created in the repository

- The struct for a waveform sample should be defined in the I/O header

- The I/O header and source should be coded and tested.

- The Waveform header and source should be completed one required output at a time and tested

- Main should be completed and tested

- The extension goals should then be implemented, with a full plan to be later constructed.

Its worth noting that before each code phase pseudocode will be made.

In terms of how I plan to develop beyond the first cycle, I am to run the project in sprints of goals.

## Code structure diagrams

Fig 1. File access orders/Logical flow

Fig 2. Repo Organisation chart

Fig 3. Header functions

Note that RMS voltage calculation function will also check for compliance, and that they are per phase where appropriate.

## Function descriptions

It is important to clearly define what each function of the program will do:

### I/O functions

writeOutput

- A function to provide the output to the user in the form of a text file

- Inputs: Output values of waveform analysis functions

- Outputs: One .txt file

readInput

- A function to read the .csv file from the user

- Inputs: A file directory to the .csv file

- Outputs: An array of all the waveform samples

### Waveform analysis functions

calcRMSVoltage

- A function to calculate the RMS of each phase of each wave using the :

Vrms = sqrt( 1/n (Vs^2) )

- Inputs: Number of samples int, voltages double

- Outputs: RMS voltage values double

calcPeakToPeak (calcP2P)

- A function to calculate the peak to peak of each phase using A=2sqrt(2)Vrms

- Inputs: RMS voltage values double

- Outputs: Peak to peak values (Amplitude) float

calcOffset

- A function to calculate the DC offset of each phase using sum / samples

- Inputs: Number of samples int, voltages double

- Outputs: DC offset values float

detectClipping

- A function to check if values are outside of tolerance (324.9V)

- Inputs: Voltages double

- Outputs: Number of clipped values int

### Functions implemented later

This section of function descriptions is for those that are implemented after I began implementing the other functions, as I did not know I needed them initially, but still within the base scope.

storageSetup

- A function to allocate memory to the heap for output values, and set all DCOffsetValues to 0 as they are additive

- Inputs: Pointers to all the storage for output values

- Outputs: Allocated memory

calcAll

- A function that uses all calculations at once

- Inputs: Addresses of where to store data

- Outputs: Processed data

cleanUp

- A function that takes in all the allocated memory and cleans it

- Inputs: Addresses of where the data is stored

- Outputs: Nothing! No memory left allocated :)

All of the diagrams currently needed are done.

## Pseudocode

The best place to start is to define the header files. Looking at the data it appears that all values to be read from the csv will need to be doubles as they have more than 7 decimal digits.

### waveform.h

if not defined

define

waveform struct as{

double timestamp

double phaseAVoltage

double phaseBVoltage

double phaseCVoltage

double lineCurrent

double frequency

double powerFactor

double thdPercent

} as waveform

double computeRMS(waveform waveformLog[])

double computeP2P(waveform waveformLog[])

double computeDCOffset(float phaseRMSVoltages[])

double computeClipping(waveform waveformLog[])

float detectClipping(waveform waveformLog[])

end definitions

The struct is done in this manner as it will be the easiest to read the .csv file into as the layout matches the rows provided. Doubles have been used in the raw data to maintain resolution which can be reduced at a later data if needed.

For the io header file only the input and output functions need to be defined.

### io.h

START

if not defined

readInput function

writeOutput function

END

For the io source file, we need to be able to read the incoming file, create an array  to store these values in, and write an output file based on the processed data.

### io.c

START

include stdio

include waveform

readInput function (takes in file path):

open file as a File type variable as a pointer to read

check that the file was found

create list of waveform structures that is x samples long 		using malloc

while fgets continues to not return null:

use strtok to write each time sample and phase voltage to	respective places in struct in array of structs

close file

writeOutput function (takes in all statistics):

i = 0

while file not written:

check whether file exists by opening to read and checking if 	cursor returns null then close

if doesnt exist:

create file of name outputStatistics(i).txt

written

else:

i++

write to file all statistics (Formatting to be determined later)

close

For the waveform source file, we need to be able to calculate all the values defined in the requirements document.

### waveform.c

include waveform.h

include math and stdio

computeRMS():

create temp variable for phase A, B and C votages ^ 2

for all samples in waveform log:

add phase voltage squared to appropriate temp 		variable to create sum of voltages ^ 2

set RMS output array values equal to sqrt(sum of 			voltages^2/number of samples)

return

computeP2P():

Take corresponding RMS value for the phase and multiply by 2sqrt(2), set equal to corresponding value in P2P values

return

computeDCOffset():

sum all voltages for each phase

divide by sample count

set output equal to corresponding value in DCOffsetValues 		arrays

return

detectClipping():

create temp variable for each phase

for all samples in waveform log:

get absolute values of each phase voltage

if a abs phase voltage is greater than 324.9

increment corresponding clipping count

### main.c:

START

readInput()

calcAll()

writeOutput()

END

A couple notes on pseudocode

These are not concrete plans by any means, and I expect the implementation to change a large amount as where I have little experience in C programming other than the tutorials, it is difficult for me to understand exactly what can do what.

The method of calculating peak to peak voltage is based on my knowledge from physics at A-Level, as the RMS is intrinsically tied to magnitude as they are both measures of amplitude.

Below are several notes I made as I went on how the code implementation was going. Not all is included here and some is moved into the version history.

## Implementation

Notes on actual implementation io.c:

One of the first problems I ran into was checking if the file i was looking for exists yet. This link is what helped me understand how to carry out this kind of operation. Also, I was initially planning on using strtok in my code to split the data, however given that the data will always come in a given format I have used sscanf(). This could change later given I have the time. filepath was initially intended to be passed into the function as a parameter, however I found it easier to contain it within the readInput subroutine, and it also had the benefit of making it easier to read main.c.

Notes on implementation of waveform.c:

I found that sample count needs to be supplied to each calculation process to make it easier to iterate. I also changed the input of timestamp to float to save memory. Added storage arrays that are allocated to the heap so that data is better managed. Added a subroutine that runs all other calculations functions to make main cleaner. Added a subroutine to malloc storage arrays for outputs. Added loop to zero clippingCounts as they were returning garbage values.

Furthermore, near the end of my final tweaks I decided to change P2P values calculation to being Max – Min as this would be closer to the requirements of the brief as opposed to trying to cut corners using a rough calculation.

As I was coding, I was also making continuous commits to the remote. Below is one example of how I carried out these, and also a snapshot of the git history from github.

## Getting a working program

In order to make the program compile you have to cd into the correct folder and then run 2 CMake commands. The correct configuration for me was

cd PowerQualityWaveformAnalyser

cd build

cmake .

cmake –build .

This was my standard compiling using my CMakeLists.txt. I made sure to use C99 standard to comply with the brief and set my source and include directories accordingly.

As I went along, I ran into many issues. The main issue that I continuously ran into was my understanding and hence use of pointers. I struggled with the use of & vs *, especially when passing things such as my array of structs, as double pointers were required. More examples include mixing * and [] in my storage arrays, forgetting * where they were needed when defining variables, and forgetting to use & to write back to the original data. Again, much of my knowledge was gained from GeeksForGeeks.

I also had struggles with not declaring the storage for the output variables in the correct scope, as they were initially being defined in a subroutine, but they needed to be defined in the main, which became abundantly clear after I realised this.

I was also incorrectly using = in for loops as opposed to < in cases such as i = sampleCount -> i < sampleCount, which in turn caused the program to timeout as it indefinitely iterated seemingly.

My main technique used to understand where errors were coming from was to print statements to the console after each major stage of a subroutine or on finishing one, allowing me to slowly refine where the problems were coming from. One example of this was the program kept closing after reading the inputs and going into calcAll, so seemed to be something to do with passing in waveformLog. I added print statements into the sequence to confirm this problem, which revealed that it would keep printing status reports all the way through the storageSetup routine, but would fail to print into calcAll. This revealed that something must be wrong with the data going into calcAll. It turned out that I was incorrectly only using one * as opposed to ** to provide a pointer to a pointer along with an & when passing the parameter, as this would allow me to alter the data at the original location. Below is an example of how I checked that each sample was being written into waveformLog as there was another stage at which the loop was not correctly including all the elements.

Below is the first output I obtained that resembled vaguely correct data.

After this only minor tweaks were needed, i.e adding more line spacing to the output file and making sure I set the clipped values to 0 when initialised. The accurate and final output for the provided csv file is below:

## Unit testing

One of the key processes I carrier out was the testing of the units once a correct output was obtained. This is basically the process of testing each individual subroutine of the program to ensure that they each provide isolated correct results. I will show a few examples below. The outputs for these examples can be found in the test folder:

readInput(waveform **waveformLog):

waveform **waveformLog is always a set input that cannot be changed, but the parameter we care about seeing if it provides the right output is the user input of directory:

| ID | Test | Expected Output | Actual output |
| --- | --- | --- | --- |
| 1 | Input nonsensensnensen (Garbage value) | "Error, could not open file. Please try again" | Same as expected |
| 2 | Input C:\Users\james\Documents\GitHub\programming_for-engineers\PowerQualityWaveformAnalyser\power_quality_log.csv (Actual csv path) | Count of numbers 0 through 999 printed to terminal as this shows it has passed the file check | As expected |
|  |  | Set of data printed to terminal that is identical to the csv as this shows it is reading accurately | As expected |

The outputs were correct to expected, but it did make me realise that I missed a \n.

computeRMS(waveform *waveformLog, double *RMSValues, int sampleCount):

The function to calculate RMS should always provide 3 outputs of approximately 230V. There are few variables in this function as it is just a calculation. This is the same for many of the other calculations. We can now use the output log from readInput as we have validated its functional correctness.

| ID | Test | Expected Output | Actual Output |
| --- | --- | --- | --- |
| 1 | Run with the standard output waveformLog from readInput | PhaseA,B,C = ~230 | PhaseA,B,C = ~230 |

computeDCOffset(waveform *waveformLog, float *DCOffsetValues, int sampleCount):

This should again be simple as it is just calculations.  I’m expecting a fixed output for each of approximately 0V.

| ID | Test | Expected Output | Actual Output |
| --- | --- | --- | --- |
| 1 | Run with the standard output waveformLog from readInput | DC Offset A,B,C = 0 | DCOffset A,B,C = 0 |

computeP2P(waveform *waveformLog, float *P2PValues, double *RMSValues,int sampleCount):

| ID | Test | Expected Output | Actual Output |
| --- | --- | --- | --- |
| 1 | Run with the standard output waveformLog from readInput | Peak to Peak A,B,C = 650 | Peak to Peak A = 0 B, C = 281 |

It turned out that after I had changed to using the Max – Min calculation instead of 2sqrt(2) as a multiplier, I was forgetting to increment the pointer in my calculations, hence why my output was just the initial values. This was then resolved and correct output obtained. The tests for other subroutines were also tested, results either being in the output data files or screenshots in the test folders.

detectClipping(waveform *waveformLog, int sampleCount, int *clippingCounts):

| ID | Test | Expected Output | Actual Output |
| --- | --- | --- | --- |
| 1 | Run with the standard output waveformLog from readInput | Peak to Peak A,B,C = 20 | Peak to Peak A, B, C = 20 |

Functions such as writeOutput and storageSetup are assumed to be working as they require to be integrated with the results of the outputs of the other functions or for storage setup there is no clear way to see what  has been assigned to the heap other than the presence of results from other functions.

All expected values are provided by the material given on blackboard, and has been cross referenced with batch processing  that has been carried out.

## Integration testing

The last stage of validation was to test the program as a whole. To do this I simply put in an accurate directory and an inaccurate one to the .exe. As expected, it was able to take in the input, discard it and ask for another input if it was incorrect, and process the data, providing an output .txt file.  If an empty file is queried, then it will provide this output:

The –nan(ind) must come from dividing by 0 on the sample count as when removing it from one of the calculations it yielded an output of 0 instead, so it isn’t the square root. Functionally a simple fix would be to have a different set of functions after checking is samples were 0 or not, however in the spirit of being efficient I believe it is best to leave this and simply put a note in the readme about this output.

The desired output for an accurate set of data was also achieved, which can be found in the writeOutput test folder or under the statAnalysis folder.

## How it has changed and a conclusion

As my code has quite notably changed since the start of the project, I want to just go over the biggest changes:

- Added multiple functions; cleanUp, storageSetup, calcAll etc.

- Changed many of the input parameters as I decided to start passing sampleCount into many  more of my subroutines as I did not initially clearly thing that they needed to be added.

- The actual logic of many of my functions, too much to say here

Given there were a lot of changes, there are lot of things I would do differently another time.

Firstly, I would more clearly layout my pseudocode. I was happy with my diagrams, however if I had put more time into my pseudocode it would have meant that I would have had an easier time implementing the newer blocks of code. It also would’ve meant that as I would understand each functions needs more, my diagrams could have been slightly more accurate.

Another aspect I would like to improve would be my version discipline, as while is used both version history in the banners and version control, they were often disjointed and sometimes skipped changes altogether. I would also have liked to more clearly work on one are of the program at a time as a side note to this.

Furthermore, I wish that I had decided to separate my files into more sections earlier, as I realised too late that logically the analysis functions should probably be outside of waveform and in their own header/source file combination.

The main things that I am happy with are the function definitions that I did do, and that I managed to keep as much code as possible outside of main.c, allowing it to be incredibly readable. I am also happy with many of my comments.

In the end I have also chosen to not include the batch processing and bitwise as they are too complicated for the timeframe I gave myself, so a bit of an improvement in time management could be seen.

Overall, I believe that I have preformed well, whilst still learning a lot from my first ever full compiled CMake project.

## Additional requirements

Anything below this point is beyond the standard spec and hence may be unfinished at the point of being handed in, but regardless should still be considered.

In the project brief document, there were also stretch goals for this project, they are listed below:

| Extension | Description | Why it is harder |
| --- | --- | --- |
| Statistical analysis | Compute standard deviation and variance of each phase voltage | Requires a two-pass algorithm; you must explain the mathematical derivation in your logbook |
| Batch processing | Accept a directory path and process every .csv file in it, producing one report per file | Requires dynamic file list management, multiple malloc/free cycles, and robust error handling |
| Sorting | Sort samples by voltage magnitude using a pointer-based sorting function | Must implement the sort yourself (no qsort); choose and justify your algorithm in the logbook |
| Bitwise status flags | Encode the health status of each phase as a bitmask in a uint8_t field (e.g. bit 0 = clipping, bit 1 = out of tolerance) | Tests understanding of bitwise operations; you must explain the flag layout in the viva |

As the framework of the code is already created, it will be easier to go through the requirements here one by one as this way I can better identify where is best to slot in them into the code. By the looks of it, the best one to begin with is sorting.

### Sorting

Simply put, we need sort the set of samples based on the voltage amplitude. As each samples has 3 amplitudes of voltage, we will only be able to sort by one, so logically it makes the most sense to sort by phase A voltage.

Simply put there will be a function called sortSamples, following the basic idea:

sortSamples

- A function to sort the list of samples based on phase A voltage

- Inputs: Unsorted list of samples

- Outputs: Sorted list of samples

Now the problem with sorting is that there is no one best way to go about it, so a bit of research is needed. As I was familiar with the website I went to GeeksForGeeks to look over some simple sorts. Fortunately by looking over the sets of comparison based sorts provided, it seems the quick sort, one that I’m familiar with, is one of the lowest complexities whilst retaining a good worst case scenario. On average it is O(nlog(n)), and worst case it is O(n^2). The reason that this is preferable to other sorts such as the merge sort isthat even though sorts like this have a better best case scenario, it requires a lot more memory to be implemented, so in the interest of being as memory efficient as possible a quick sort is better. It also out-does a merge sort in the manner that as the votlage data generally follows an approximately random pattern for the purpose of the sort, the worst case complexity is unlikely to even be hit. It is also the one that I did in A-Level algorithms.

Simply put this type of sort defines a pivot value, and will compare each number from itself to the next fixed value to the right of it, and swap numbers to either side based on whether they are higher or lower. To put this into pseudocode:

quickSort(array to be sorted):

get the max and min

If the low is still before the high:

set the current high as the pivot for the array

swap the elements higher than this pivot to being on the 		left of the pivot

then swap the element just after all the elements moved 		o the left of the pivot and the pivot, returning the 			index of this

call this function again in the ranges of low to i-1 and 	i+1 to high

Again, much of the logic for this comes from the geeks for geeks website.  In terms of pointer logic, it should be like readInput as I am aiming to change the set of waveforms as opposed to just getting data from it, so using ** to get a pointer to a pointer.

To test the written code which fortunately worked first time thanks to the massive help of the above mentioned website and my prior experience of pointers in this program letting me go smoothly, I simply printed it to command line.

## Statistical analysis

The statistical analysis required for each phase is that I must calculate the standard deviation and the variance.  The equations can be found online in many places, in this case I’ve taken them from GeeksForGeeks.

𝜎2=1𝑛𝑖=1𝑛𝑥𝑖−𝜇2

Omega being the standard deviation, so standard deviation is the square root of variance.

The reason the standard deviations is useful is because traditionally if you wanted to measure spead you could use the difference from the mean to each of the values:

1𝑛Σ𝑥𝑖−𝜇

However the issue with this is that the positive and negative deviations from the mean would cancel out, so if we add some sort of square we can insure that all deviations are considered. This gives us the variance equation:

𝜎2=1𝑛Σ𝑥𝑖−𝜇2

This can now be returned back to the original dimension of deviation as we are still a square factor out, so simply

𝜎=1𝑛Σ𝑥𝑖−𝜇2

Giving us the equation for standard deviation.

So we know that we need the mean value for each phase of the wave to calculate the variance, which is the same as the DC offset, so we must do this after calculating the DC Offset.

In pseudocode:

calcVarDev(waveformLog, DCOffsets, varianceValues, standardDeviationValues, sampleCount):

for every sample in waveformLog:

sumA,B,C += phaseA,B,C voltage – phaseA,B,C DCOffset

varianceValuesA,B,C = sum / sampleCount

standardDeviationValues = sqrt(varianceValues)

Implementation notes:

When implementing my code, it compiled correctly, however I didnt seem to be getting an output. To understand this, I added some print statements with a sleep. I put this initially at the end of calcAll where it was called and it did not show up. I then moved this to just before the statAnalysis call and it did show up so we know it’s in there. I then put it after ever count of the pointer, and it provided this for approximately 1000 times so it was doing that part fine:I then put it after the loop and I stopped showing up, so I know its something to do between the end of the loop and the returnI then put one just after the loop and one after the divisions. It only returned one, so it is stopping at the divisions. I then removed the divisions from the top one and put another print, but it did not show up. Following this I tried to print just the sumVal /sampleCount which printed, so it must be to do with the setting equal to variance values. I then changed all types to double but no luck. Consequently, I decided to compare it to an already implemented function and realised I forgot to malloc my storage arrays, which was a simple fix in the end. This is another example of the kind of debugging procress that I go through. This did take a lot of time given going back and forth between doubles and floats, but I got there in the end.

statAnalysis(waveform *waveformLog, float *varianceValues, float *standardDevValues, float *DCOffsetValues, int sampleCount):

| ID | Test | Expected Output | Actual Output |
| --- | --- | --- | --- |
| 1 | Run with the standard output waveformLog from readInput | Var = 52812 Dev = 229 | Var = 52812 Dev = 229 |

Note that here I have assumed readInput to continue to throw no errors as it has already been tested with minimal changes to implement the new statistics.

## Batch processing

This requirement mandates that I should be able to filter a whole directory with just one input. The main issue with this is that at the minute it will only check one level of files, and it will only do one file in the given directory. It will also break given that a folder is provided.

The spec indicates that it should either be given a folder or a file, so I need a way to check what type of file/folder I have been given. It is also worth checking that all files are .csv for those that will be analysed. So how it should work:

- Take an input directory

- Check if it is a folder or file

- If it is a folder open it and check how many files are inside of it

- Then for each file start processing it as before. If the file type isn't csv state that it isn't to the terminal then continue to other files

The request for directory will need to moved to outside of the readInput loop to prevent a major logical rework.

Blocks of code to be made following thought process:

- requestDirectory

- isFolderOrFile

- updated readInput with directory as a parameter

requestDirectoty should be simple as it has basically already been implemented in the old version of readInput so I won’t repeat myself there.